Mid-Term Examination

Analog/IC Filter Design (ELEC 441/6081)

Electrical and Computer Engineering Department

Concordia University

February 28, 2007

Instructor: Dr. R. Raut				Time: 75 minutes

Q.1: In the figure below all voltage and currents are small ac signals. The elements Y1, Y2 can be chosen to produce different signal processing functions pertaining to electronic filters.

- What will be your choice for Y1, Y2 so that the function Vo/VS  is a second order transfer function in ? Prove your point by an analytical expression for Vo/VS .

- If in the ideal case of Ri0, Ro0, and RmY1>>1,  show that Vo/VS  becomes an inverting integrator function with Y1=sC1

- With the same conditions as in (b) above and further that Y2=sC2, and Y1=1/R1, show that io/VS represents a differentiator function.

Q.2: You are required to design a BPF with the pass-band extending from =105 rad/sec to =4105 rad/sec. The filter has equal ripple characteristic in the pass-band with peak-to-peak value not exceeding 1 dB. At =15.263105  rad/sec, the response must be at least 60 dB down relative to the pass-band.

- Synthesize the normalized low-pass filter function associated with the intended band-pass filter.

- The figure below shows an L,C realization of the low-pass filter function in (a). Based on this, design the band-pass filter as specified in Q.2 above.

Q.3: Consider the band-pass filter network of figure below. It combines both positive and negative feed-back around an ideal OA.  The voltage transfer function is given by:

, with .

Assume K=1. Design the element values to meet the specifications: fp=10 kHz, Qp =10 and C=0.1 uF each.

Filter Function Tables

A.1: Coefficients of denominator polynomial, in the form sn+a1sn−1+a2sn−2+...+an−2s2+an−1s+1, for Butterworth filter function of order n , with pass-band from 0 to 1 rad/sec+.

| n | a1 | a2 | a3 | a4 | a5 | a6 |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 1.4142 |  |  |  |  |  |
| 3 | 2.0000 | 2.0000 |  |  |  |  |
| 4 | 2.6131 | 3.4142 | 2.6131 |  |  |  |
| 5 | 3.2361 | 5.2361 | 5.2361 | 3.2361 |  |  |
| 6 | 3.8637 | 7.4641 | 9.1416 | 7.4641 | 3.8637 |  |
| 7 | 4.4940 | 10.0978 | 14.5918 | 14.5918 | 10.0978 | 4.4940 |

A.2: Coefficients of denominator polynomial, in the form sn+a1sn−1+a2sn−2+...+an−2s2+an−1s+an, for Chebyshev filter function of order n , with pass-band from 0 to 1 rad/sec+.

| Pass-band ripple Ap | n | a1 | a2 | a3 | a4 | a5 | a6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2.863 |  |  |  |  |  |
|  | 2 | 1.425 | 1.516 |  |  |  |  |
| 0.5 dB | 3 | 1.253 | 1.535 | 0.716 |  |  |  |
| ε=0.3493 | 4 | 1.197 | 1.717 | 1.025 | 0.379 |  |  |
|  | 5 | 1.1725 | 1.9374 | 1.3096 | 0.7525 | 0.1789 |  |
|  | 6 | 1.1592 | 2.1718 | 1.5898 | 1.1719 | 0.4324 | 0.0948 |
|  | 1 | 1.965 |  |  |  |  |  |
|  | 2 | 1.098 | 1.103 |  |  |  |  |
| 1.0 dB | 3 | 0.988 | 1.238 | 0.491 |  |  |  |
| ε=0.5089 | 4 | 0.953 | 1.454 | 0.743 | 0.276 |  |  |
|  | 5 | 0.9368 | 1.6888 | 0.9744 | 0.5805 | 0.1228 |  |
|  | 6 | 0.9282 | 1.9308 | 1.2021 | 0.9393 | 0.3071 | 0.0689 |
|  | 1 | 1.308 |  |  |  |  |  |
|  | 2 | 0.804 | 0.637 |  |  |  |  |
| 2.0 dB | 3 | 0.738 | 1.022 | 0.327 |  |  |  |
| ε=0.7648 | 4 | 0.716 | 1.256 | 0.517 | 0.206 |  |  |
|  | 5 | 0.7065 | 1.4995 | 0.6935 | 0.4593 | 0.0817 |  |
|  | 6 | 0.7012 | 1.7459 | 0.8670 | 0.7715 | 0.2103 | 0.0514 |

A.3: Coefficients of denominator polynomial, in the form sn+a1sn−1+a2sn−2+...+an−2s2+an−1s+an, for Bessel-Thomson filter function of order n+.

| n | a1 | a2 | a3 | a4 | a5 | a6 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 |  |  |  |  |  |
| 2 | 3 | 3 |  |  |  |  |
| 3 | 6 | 15 | 15 |  |  |  |
| 4 | 10 | 45 | 105 | 105 |  |  |
| 5 | 15 | 105 | 420 | 945 | 945 |  |
| 6 | 21 | 210 | 1260 | 4725 | 10395 | 10395 |

+ R. Schaumann et al, “Design of Analog Filters- Passive, Active RC, and Switched Capacitor”, Prentice-Hall Inc., © 1990