Mid-Term Test#1

Analog/IC Filter Design (ELEC 441/6081)

Electrical and Computer Engineering Department

Concordia University

February 27, 2008

Instructor: Dr. R. Raut				Time: 75 minutes

Q.1: The circuit below is a low-pass filter with AP =0.25 dB and ωp =1 rad/sec. Modify the circuit so that

(a) The pass-band becomes 10 kHz.

and 	(b) The load resistance becomes 75 ohms.

Figure 1

Q.2:  Find the transfer function of a high pass filter which has

a) an equiripple pass-band for f> 15kHz with Ap = 0.5dB.

and 	b) a monotonic stop-band for f<7.8 kHz with Aa= 35dB.

Q.3:  A low-pass filter of order 5 with maximally flat magnitude approximation has a loss of 1 dB at the pass-band edge frequency of 1 kHz.

(a) What will be attenuation produced by this filter at 10 kHz?

(b) What will be the order of the filter if we require a loss of 40 dB at 40 kHz?

Q.4: The following network represents a low-pass filter. Derive the expression for the VTF (i.e., V2(s)/V1(s)) of the network. The OA is ideal.

Figure 4