Mid-Term Examination

Analog/IC Filter Design (ELEC 441/6081)

Electrical and Computer Engineering Department

Concordia University

March 28, 2007

Instructor: Dr. R. Raut										Time: 75 minutes

Q.1: You are required to use a cascade of gm-C filter sections to implement a CHEB-filter with Ap=0.3 dB, fc=10 MHz, Aa=22 dB, and fa=25 MHz. The gain at DC is to be unity.

(a) Show a system diagram for your implementation, and

(b) Calculate the design values (i.e., gm and C).

Q.2: You are required to design the notch filter with transfer function H(s)=s2+1.4212×105s2+1004.2s+6.9833×105 using switched capacitor technique. The given clock frequency is 8 kHz.

- Determine the sampled-data transfer function H(z) that is to be designed using bilinear s to z transformation.

- Show a possible SC network that could be used to implement the switched capacitor filter.

Q.3: Consider the implementation of a high-pass CHEB filter with 1 dB ripple in the pass-band for f1400 Hz. In the attenuation band, the attenuation is 35 dB for f375 Hz. The associated normalized low-pass LC ladder filter is shown below.

Produce an active RC high-pass filter network using the above LC ladder as the starting point.